# OpAmp Divided Feedback

# OpAmp Divided Feedback

| Rlower=1000 | [Ohms] |
| --- | --- |

| Rupper=4000 | [Ohms] |
| --- | --- |

| IB=0 | [A] |
| --- | --- |

| IOS=0 | [A] |
| --- | --- |

| VOS=0 | [V] |
| --- | --- |

| Vin=1 | [V] |
| --- | --- |

| VOUT=Vin+VOS⋅1+RupperRlower+IB⋅Rupper+IOS2⋅Rupper | [V] |
| --- | --- |

Extreme Value Analysis

Query: VOUT=

| Parameter | Min | Max |
| --- | --- | --- |
| Rlower | 998 | 1002 |
| Rupper | 3996 | 4004 |
| IB | -600⋅10-9 | 600⋅10-9 |
| IOS | -100⋅10-9 | 100⋅10-9 |
| VOS | -500⋅10-6 | 500⋅10-6 |

Min: 4.9829

Max: 5.0171

Sensitivity: - R_lower_as_variable: 46.8% - R_upper_as_variable: 23.4% - V_OS_as_variable: 14.6% - I_B_as_variable: 14.0% - I_OS_as_variable: 1.2%